# Heat Press Class (First Draft)

(dye sub 101 prerequisite)

Intended Use:

- Not for commercial use. If your profit model relies on the UPTIME of DMS Tools, this is unacceptable use.

- If your use is preventing multiple members from using the machine- this is beyond intended use.

- Use will be monitored by camera & log.

- Unlogged use will be banned

  - If you merely forgot, message Vector@dallasmakerspace.org with the day/time/approximate # of presses.

- Do not tamper with 3d Printed box covers. Tampering with box covers will result in permanent tool ban.

## What Does it Do?

Large, flat iron. It is the final step in many pint processes.

- Direct to Film (DTF) Pressing

- Dye Sublimation Pressing

- Heat Transfer Vinyl Pressing

- Also used for applique and interfacing heat fusible fabric.

Yes, but what can I make with it?

- T-shirts, hoodies, apparel

- Signs/Banners

- Custom Fabric Designs

- Press-n-cut clothing

- Cosplay Body Suits

- Fine Art Canvas Prints

- Photographs on Metal

- Balloons!

## What Materials?

Items under 1/8” thick. MUST BE CLEAN

Examples:
Fabric, T-shirts, thin metals (

# Set Up:

- Hook up:

  - Pneumatic

  - Power

- Set Heat

- Set time

## Lay Out Material:

### Dye Sub/DTF:

- Lay out paper under if any material may get on rubber mats

- Lay out as flat as possible

- Dye sub/DTF film face down

- Cover with paper

- Press both green buttons to start first press

- Repeat when second Platten is done

### Fabric Fusing:

- Set temp/time according to interfacing directions.

- Fusing must be smaller than fabric

  - When in doubt, put paper down first

- Place fabric ‘good’ side down

- Place interfacing adhesive (bumpy) side towards fabric.
If double sided interfacing, place second piece of fabric ‘good’ side up.
Cover all with paper.

- Press both Green buttons once to activate press.

# Resources:

## Dye Sub:

- Print in-house, carefully paste together.

- Alpha Graphics can mail off image.

  - Image must be 300dpi. Print out a small sample on normal printer to be sure it looks good.

  - Be sure to flip image

- WeSubNStuff

  - Process is in the works, local large format (44in) dye sub printer

## Heat Transfer Vinyl:

- Must take vinyl cutter class to learn to cut vinyl

  - Apply to surface, press at time/temp

## Direct To Film:

### DTForder.com
https://dtforder.com/products/gang-sheet-1

  - 22x24 is cheapest per linear foot, use gang sheet builder, 
Sheet already has margin, be sure to UNCHECK the margin!

  - Where is it? 10MIN From DMS. No ship, 4hrs to print.

- DTF Dallas

  - Better quality, slightly more expensive

  - https://dtfdallas.com/products/custom-gang-sheet?variant=47278795555104